Computers & Industrial Engineering 64 (2013) 917–928

Contents lists  available at SciVerse ScienceDirect

Computers & Industrial Engineering

j o u r n a l  hom ep ag e:  www. el se vi er .com /l ocate / ca ie

Joint location and  dispatching decisions for Emergency Medical Services

Hector Toro-Díaz a, Maria E. Mayorga a,⇑, Sunarin Chanta b, Laura  A. McLay c

a Department of Industrial Engineering, Clemson University, Clemson, SC 29634, USA

b Department of Industrial Management, King Mongkut’s University of Technology North Bangkok (Prachinburi Campus), Prachinburi, 25230, Thailand

c Department of Statistical Sciences & Operations Research, Virginia Commonwealth University, Richmond, VA 23284, USA

a r t i c l e    i  n f o

Article history:

Received 27 July  2012

Received in revised form 17 December 2012

Accepted 14 January 2013

Available online 4 February 2013

Keywords:

Location/allocation in healthcare

Hypercube model

Genetic algorithm

a b s t r a  c t

The  main purpose of Emergency Medical Service systems is to save lives by  providing quick response to emergencies. The  performance of these systems is affected by  the location of the ambulances and their allocation to the customers. Previous literature has suggested that simultaneously making location and dispatching decisions could potentially improve some performance measures, such as  response times. We  developed a mathematical formulation that combines an  integer programming model representing location and dispatching decisions, with a hypercube model representing the queuing elements and con- gestion phenomena. Dispatching decisions are  modeled as  a ﬁxed priority list  for  each customer. Due  to the model’s complexity, we  developed an  optimization framework based on  Genetic Algorithms. Our results show that minimization of response time and maximization of coverage can  be  achieved by  the commonly used closest dispatching rule. In addition, solutions with minimum response time also yield good values of expected coverage. The  optimization framework was able to consistently obtain the best solutions (compared to enumeration procedures), making it suitable to attempt the optimization of alter- native optimization criteria. We  illustrate the potential beneﬁt of the joint approach by  using a fairness performance indicator. We  conclude that the joint approach can  give  insights of the implicit trade-offs

between several conﬂicting optimization criteria.

2013 Elsevier Ltd. All rights reserved.

1. Introduction

Emergency Medical Service (EMS) systems are  a public service that provides out-of-hospital acute care  and transport to  a  place of deﬁnitive care,  to patients with illnesses and injuries that consti- tute a medical emergency. The ultimate goal  of EMS systems is to save  lives.  The ability of these systems to do this effectively is im- pacted by  several resource allocation decisions including location of servers, districting of demand zones and dispatching rules for the servers. Common objectives are  minimizing the mean response time and/or maximizing coverage. The relationship between mini- mizing response time and improving survivability has   been re- ported by  several works such as  Sanchez-Mangas, García-Ferrer, de   Juan,   and  Arroyo (2010)  and  McLay   and  Mayorga  (2010,

2011). A demand zone is  said   to  be  covered if  there is  at least one  facility within a predeﬁned distance/time threshold from the demand zone. The concept of coverage is related to the availability of a satisfactory facility rather than the best possible one  (Farahani,

⇑ Corresponding author. Address: Department of Industrial Engineering, Freeman

Hall, Box  340920, Clemson, SC 29634, USA. Tel.:  +1  864 656 6919; fax: +1  864 656

0795.

E-mail addresses:  htoro@clemson.edu  (H.   Toro-Díaz), mayorga@clemson.edu

(M.E.  Mayorga), snct@kmutnb.ac.th (S. Chanta), lamclay@vcu.edu (L.A. McLay).

Asgari, Heidari, Hosseininia, & Goh, 2012). Li, Zhao, Zhu, and Wyatt (2011) pointed out  that the coverage maximization approach is the most widely used by practitioners, researchers and regulators.

Traditionally, location and dispatching decisions have been ap- proached separately, even though various studies have shown that the servers’ busy probabilities (and therefore the response time and coverage, among other performance indicators) are  sensitive to the server locations and the choice of server dispatching strate- gies  (Batta, Dolan,  & Krishnamurthy, 1989; Larson & Odoni, 1981). Ambulance dispatch is the process of assigning a particular ambu- lance to  answer an  emergency call.  An ambulance dispatch policy can  be formed using various dispatch methods and there is no sin- gle  policy that ﬁts  all  systems (Li et al., 2011). The  same authors emphasized that a  dispatch  policy has   to  be  designed to  fulﬁll the particular objectives and performance indicators deﬁned by EMS providers and regulators. In  our  work we  consider dispatch policies in which there is a single list associated with each demand zone that ranks the available servers (ambulances), or a subset of them, in order of dispatch preference. This type of list is commonly referred to  as a contingency table.

The  most common dispatching policy for  EMS calls  is  rather simple in  that the closest idle  vehicle is usually dispatched to  at- tend the call  (Andersson & Varbrand, 2006; Goldberg, 2004). The rationale behind that policy is  related to  the idea of  having the

0360-8352/$ - see front matter     2013 Elsevier Ltd.  All rights reserved. http://dx.doi.org/10.1016/j.cie.2013.01.002

works on  allocation of  distinguishable servers by  Jarvis   (1981) and Katehakis and Levine  (1986) pointed out  that under light traf- ﬁc  conditions using a  myopic allocation policy (i.e.  assigning al- ways the closest available sever) will  lead to  an  optimal solution, when the objective is to  minimize the long  run average cost  (re- sponse time). For  heavy trafﬁc the same works mentioned that the optimal policy can  deviate from the myopic policy. However, even in the latter case  using the myopic policy still  might lead to solutions that are  close the optimum (Katehakis & Levine,  1986). Related literature applied to EMS systems planning included argu- ments against the closest dispatching rule as  a way  to  minimize the  response time. Arguments were made originally by  Carter, Chaiken, and Ignall (1972) and thereafter supported by  Cuningh- ame-Green and Harries (1988) and Repede and Bernardo (1994). In the referred works the locations of the servers are  assumed to be  known. We  have not  found references addressing the relation- ship between a myopic dispatching policy and expected coverage. There is usually a trade-off between response time and coverage, so that improving one  of them implies a sacriﬁce in the other.

In this work, ﬁrst we  present a mathematical model that inte- grates the location and dispatching decisions for  an  EMS system. It is a non-linear mixed integer optimization model in which even generating some of  the equations is  computationally  intensive, therefore making it  hard to  solve. The  Hypercube model is  used providing an   exact model of  the  stochastic queuing dynamics. The  mathematical model is  accompanied by  the analysis of  ran- domly generated small instances whose purpose is twofold: (i) gi- ven   the small size   it  is  possible to  fully  enumerate all  feasible solutions hence also  identifying the optimal, that can  be used later for comparison purposes against faster/smarter solution strategies than enumeration; and (ii)  after solving a  variety of  random in- stances it  is  also  possible to  point out   some general trends ob- served in  the optimal solutions (with respect to  response time and coverage). Second, we  present an  optimization framework to solve the joint location and dispatching problem based on Genetic Algorithms (GAs).  We  present a  heuristic solution procedure to solve the exact model of the system. Our  work is  different from previous approaches to  the problem, for  although we  assume the general form of the dispatching policy, as  a ﬁxed preference list, we  do not  assume a priori any  particular dispatching order (based on  distance, for example). Instead, we  model the location and dis- patching decisions in a single mathematical model, and develop an optimization framework for  its  solution. In fact,  since a district is the union of the demand zones assigned to  a particular server, it can  be said  that an indirect result of our  model is also  a districting strategy: for each available server, all the zones having it as its ﬁrst preferred server would form the server’s district.

Our ﬁndings are  that in fact the common dispatching rule based on the closest available server leads to the best solutions when the objective is minimizing the mean response time and locations are optimized simultaneously. Conversely, if the objective is maximiz- ing  expected coverage, then  the optimal solution could deviate from the use  of  the closest dispatching rule. However, the best solutions based on  coverage offer   an  increase of  that indicator (with respect to the coverage attained by minimizing the mean re- sponse time) that is rather small (3.15% average increase – 95% CI:

2.75–3.55%) compared  to  the sacriﬁce in  response  time  (65.2% average increase – 95% CI: 56.33–74.24%). Although these numbers correspond to the average results for the small instances, bigger in- stances showed similar behavior. The optimization procedure pro- posed has  consistently obtained good  solutions, i.e. within 1% gap compared to the best solutions obtained by full or partial enumer- ation procedures, which are  computationally more intensive.

While our  main goal  was  the development of the optimization framework for the solution of the joint location/dispatching prob-

grated approach when using the two most commonly used criteria, namely response time and expected coverage. Thus  we  considered two additional criteria related to fairness, and we used one  of them to  illustrate the potential beneﬁts of the joint approach. In partic- ular we  tested the variance of the individual response times as  a measure of fairness from the point of view of the users of the sys- tem (demand zones). We  found that in  this case  using a  myopic policy would result in a potential deviation from the optimal policy aimed at reducing disparities, as measure by the variance of the re- sponse times. We  also   illustrate the trade-offs among the pre- sented optimization criteria.

The  rest of the paper is organized as  follows. In Section 2 we provide the presentation of the problem as well  as a review of re- lated literature. Next,  in Section 3 we  introduce the mathematical model. Section 4 presents a small case  study, as well  as a summary of  its   results  and  implications.  Section 5  provides  a  detailed description of the optimization framework based on  GAs and Sec- tion 6 introduces the case  studies to which the optimization proce- dure is  applied, as  well   as  the  results obtained. The  last   two sections, 7 and 8 are  the discussion of the results and the conclu- sions, respectively. As part of the conclusions possible extensions of the present work are  mentioned.

2. Problem presentation and related literature

In Goldberg’s review of models for deployment of EMS vehicles (Goldberg, 2004), it is mentioned that little work had been done on dispatching of ambulances. Similar opinion is shared by Lee (2011), mentioning that the contributions in  ambulance dispatching are sparse. In turn, Galvao and Morabito (2008) and Iannoni, Morabito, and Saydam (2011) mention as  an  interesting extension of  their work the  use   of  different dispatch  preference  lists,   instead  of assuming that for a given set  of locations the dispatching order is based on  the closest dispatching rule.

The most widely used dispatching rule under a ﬁxed preference scheme is  to  send the closest unit, looking to  minimize the re- sponse times (Andersson & Varbrand, 2006). The  ﬁrst argument against the use  of such a policy was  made by  Carter et al. (1972). They  present a case  where two units, A and B, have equally large areas of responsibility, but A’s area has  a signiﬁcantly higher call frequency. In  those conditions, the mean response time will  de- crease if B is allowed to  respond to  some of the calls  for  which A is the closest unit. The  result was  generalized for  cases involving more than two units by  Cuninghame-Green and Harries (1988). Repede and Bernardo (1994) also   supported the argument. The works by  Jarvis  (1981) and Katehakis and Levine  (1986) studied the  optimal allocation of  distinguishable  servers on  Markovian queuing systems, reaching a different conclusion. For a given loca- tion of the servers, so that the cost  of assigning a server to a partic- ular customer is known (the cost  in EMS planning is usually related to  the amount of time that it  takes for  the EMS system to  effec- tively respond to a call),  these two works showed that under light trafﬁc conditions (trafﬁc is  measured by  the ratio between the mean arrival rate and the mean total service rate) the use  of a myo- pic  policy always would lead to  an  optimal solution, i.e. minimiz- ing  the long  run average cost   (response time). For  heavy trafﬁc the use  of a myopic policy will  deviate from the optimal, however the deviation is rather small (2–3%).  Katehakis and Levine  (1986) used 0.38  as  an  indicator of light trafﬁc and 1.94  for  the case  of heavy trafﬁc.

We  propose a mathematical model that combines location and dispatching decisions for EMS vehicles, initially looking for optimal solutions according to  maximum coverage or  minimum response time. The dispatching decisions are  modeled as a ﬁxed preference

costumer that ranks the available servers (ambulances) ir  order of  dispatch preference. That   list  does not   change as  a  result  of changes in  the state of  the system. However, the particular unit that will  be  dispatched to  attend each call  from a  demand zone is  not   known in  advance, since the assignment depends on  the availability of the servers (system’s state) when the call is received. Katehakis and Levine  (1986) pointed out  some results from Markov Decision Theory indicating that, when the number of states of the system as  well  as  the number of actions available to  perform in every state (allocation of the servers) are  ﬁnite, it sufﬁces to  con- sider only   deterministic  policies; a  deterministic  policy is  one which, whenever the system is in particular state, the set  of avail- able  actions to perform is deterministic and depends only  of the ac- tual state (in our  case,  which servers are  busy, and which are  idle).

The servers in a typical EMS system are: (i) spatially distributed in  the region; (ii)  share the system workload due to  cooperation among them and (iii)  have different operational characteristics, such as  different preferential  regions (Galvao & Morabito, 2008). Those  characteristics have been progressively included in different approaches used for  planning EMS systems. Congestion is  also  a typical phenomena related to  EMS systems. According to  Galvao, Chiyoshi, and  Morabito (2005) the  volume of  calls   for  service may keep ambulances busy from 20% to  30% of the time.

Brotcorne, Laporte, and Semet (2003) provided a review focused on  location models and their particular application to  EMS. They classiﬁed the location models that evolved over  the past 30 years into two main categories, deterministic and probabilistic, recogniz- ing  that the most recent models were more concerned with the representation of  the stochastic nature of  the systems. Location models were also  distinguished in coverage and median type prob- lems. The  ﬁrst class  attempts to  locate the servers so  as  to  maxi- mize the fraction of the demand that has  at least one  server unit within a  predeﬁned maximum distance or  time. The  latter type minimizes the average or  total travel time/cost between servers and demand zones.

The  two seminal attempts to  develop basic coverage models were the set  covering location problem (SCLP) by  Toregas, Swain, ReVelle,  and Bergman (1971) and the maximal coverage location problem (MCLP) by Church and ReVelle (1974). Extensions to those basic models were developed later. TEAM and FLEET models, by Schilling, Elzinga, Cohon, Church, and ReVelle  (1979), considered several types of  servers; Marianov and ReVelle  (1992) improved the MCLP model. Multiple coverage of demands were considered in  BACOP1 and BACOP2 by  Hogan and Revelle (1986) and other extensions, DSM  and DDSM  were added by  Gendreau,  Laporte, and Semet (1997, 2001). The  p-median problem was   introduced by Hakimi (1964). The  use  of the p-median model in the planning and location of facilities for  EMS can  be  found in  Carbone (1974) and Carson and Batta (1990).

Basic location models are  deterministic in nature and therefore do not  represent the system accurately (Brotcorne et al., 2003; Jia, Ordóñez, & Dessouky, 2007a). Basic  coverage models might make sense when the location of facilities are  ﬁxed, but in  the case  of an  EMS system, as  soon as  a unit leaves its  home base to  attend a  request for  service, other demand points that are  supposed to be  covered by  that unit may no  longer be  covered. The  work by Snyder (2004) reviewed several models that address variations in the inputs, such as  demands and travel times, as  a  way  to  take uncertainty into account. The  same work pointed out  the impor- tance of addressing congestion. Daskin (1983) developed the max- imum expected coverage location model (MEXCLP) including the modeling of congestion elements. Hogan and Revelle (1986) devel- oped the maximal availability location problem (MALP I and II) and later Marianov and ReVelle   (1996) improved it.  Farahani et  al. (2012) present an  extensive up  to  date review on  covering prob-

a survey on  facility locations dynamics.

It was  the work by Larson (1974) that ﬁrst used queueing the- ory  elements in facility location models by introducing the hyper- cube model. Larson (1975) later developed an  approximation for the hypercube model due to  the fact  that exact calculations were prohibitive. Chiyoshi, Galvao, and Morabito (2001) pointed out, after comparing several models, that the hypercube was  the only one  with the capabilities for an accurate representation of the sys- tem. There is a variety of applications and extension of the hyper- cube model to  EMS system such as  the works by  Brandeau and Chiu  (1989), Mendona and Morabito (2001), Atkinson, Kovalenko, Kuznetsov,  and   Mykhalevych  (2008),  Iannoni   and   Morabito (2007), Iannoni, Morabito, and Saydam (2008), Galvao and Morab- ito   (2008)  and  Geroliminis, Karlaftis, and  Skabardonis  (2009), among others. It is well  documented that the hypercube model is a  descriptive tool  allowing scenario analysis, not  designed as  an optimization model. However, it  is  possible to  embed it  into an optimization framework. Batta et al.  (1989) combined MEXCLP with the hypercube into an iterative, local  search algorithm. Aytug and Saydam (2002) replaced the local  search by  a  genetic algo- rithm.  Iannoni and  Morabito (2007) as   well   as   Iannoni et  al. (2008) and Geroliminis, Kepaptsoglou, and Karlaftis (2011) have embedded the hypercube model into genetic algorithms to solve the location problem. In  this paper we  also   use   the hypercube model as it exactly models the system. While hypercube approxi- mations (Jarvis,  1985; Larson, 1975) may lead to  faster solution procedures, they do  not  provide an  exact solution. Thus,  as  men- tioned earlier, our  approach is to ﬁnd  a heuristic solution to an ex- act  problem as  opposed to  an  exact solution to  an  approximate problem. Future work, related to scalability issues of the proposed method is mentioned in Section 8.

3. Mathematical model

Our  model is different from existing literature in that we  inte- grate location and dispatching decisions into a single framework, whereas the mentioned references assumed the use  of a priori dis- patching policy, particularly based on  the closest relationship.

3.1. Assumptions

It is assumed that the system provides service to a certain geo- graphical region J that is partitioned into service regions -demand zones, cells  or atoms are  other terms that have been use  for these partitions. A given number of servers are  located at points i 2 I     J. Demands occur solely at the center of each service region by time homogeneous  Poisson requests  for  service and are   attended at exponential service rates. Larson and Odoni (1981) have shown that reasonable deviation from this last  assumption do  not  signif- icantly alter the accuracy of the model.

Each service region j generates a fraction fj  of the total demand

P

ð     j fj  ¼ 1Þ. The total demand is then k and the demand of each zone

is  kj     kfj.  A server’s primary  response  area  (district) consists of

those service regions to  which the server would be  dispatched  if available. When a request for service arrives, if the primary respon- sible server is  available, it is dispatched immediately. The  server travels to  the place of  the incident, spends some time at scene and then returns to  its  base location before being assigned to  the next request. If the responsible server is busy when a request for it arrives, another server will be assigned, following a ﬁxed priority list  with respect to the servers for each demand zone. The priority list  can  include all the servers available in the system (total back- up)  or only  a subset of them (partial backup). If all the servers are busy, the request is considered to be lost  (this typically means that

sumes that the servers are  identical and that the service time of any  response unit for any  call for service has  an exponential distri- bution with mean 1/l (This  assumption is reasonable if the travel times are short compared to the total service time, which is usually

the case  in urban areas). The service time for a call includes the set up  time, the travel time from the base to the incident location, the on-scene time, a  possible related follow up-time and the travel time back  to the base. The response time interval is the time from when an  ambulance is dispatched until it arrives at the scene.

Each  server can  be  busy or  free  (idle), generating 2N   possible

states for  the system (where N = number of  servers); the states can  be  mapped to  the vertices of a hypercube (strictly a cube for the case  of exactly three servers) named Bj(j = 1, 2, ... ,2N) of dimen- sion  N. Each vertex, or state, is denoted by an ordered set  of N one digit binary numbers taking the value of 1 if the server is busy and

0 if not  (Bj    {b1, b2, ... , bN}). It is assumed that only  one  step tran- sitions occur, i.e. two servers cannot be  assigned simultaneously. Using  the convention proposed by  Larson (1974), transitions are only  allowed between states with Hamming distance equal to  1,

where the Hamming distance dij   between two vertices Bi  and Bj

Note  that even though it was  assumed that the service rate is equal for all servers since they were identical, the general expression gi- ven  by this equation allows for different service rates for different servers. For details on  calculating kij  and lij, see  Geroliminis et al. (2009).

Constraint (6)  ensures that the sum of probabilities is equal to one.  Eq. (7)  is  the integrality constraint for  the decision variable yl ; constraint (8)  states the logical relationship between the loca- tion decision and the assignment of a location within the priority list  of a demand zone and ﬁnally, constraints (9)  and (10)  assure that there is a complete priority list for each demand zone, and that within the priority list  of each demand zone each server appears only  once. The  full  model given by  (1)–(10) represents the basic optimization problem in  which the location of  the servers and the dispatching rule for  each demand zone are  the decisions to be made. Also note that the steady-state probabilities are  auxiliary variables that change for  every full  combination of  location and dispatching decision.

ij

The  optimization problem is formulated as:

N      J

is  the number of  digits by  which the two vertices differ (or  the

‘right   angle’   distance between  two vertices of  the  hypercube). The  terms ‘‘upward’’  and ‘‘downward’’ Hamming distance, dþ  and

ij

Minimize MRT ¼ XXqnj tnj                                                                                                 ð1Þ

n¼1  j¼1

s:t :

ij , refer to  the number of  binary digits switching from 0  to 1

d

and 1 to 0. The  model of the system corresponds to  a ﬁnite-state

continuous time Markov process. Steady-state probabilities are determined from equations of detailed balance that express a con- servation of ﬂow  between consequent states. This  set  of balance

I

Xxi ¼ N                                                                                                 ð2Þ

i¼1

xi 2 f0; 1g;  i 2 I                                                                                      ð3Þ

Bi 2Enj PðBi Þ

P

equations depends on both, the location of the servers and the dis-

patching policy.

3.2. Formulation

qnj  ¼ fj

2

6

6

1    PðB2N Þ

i

n ¼ 1; .. . ; N;  j 2 J                                                ð4Þ

3

7

7

In the following formulation J represents the service regions; I

are  the potential location sites, jIj 6 jJj; N is the total number of re- sponse units (servers); tnj is the mean response time for server n to

PðBj Þ6          X

6

6

4

Bi 2 CN : dþ ¼ 1

ij

X

kij þ      X

i

Bi 2 CN : d    ¼ 1

ij

X

lij 7

7

7

5

reach region j, when available; k is the total network-wide demand (requests/unit time); fj  is  the fraction of network-wide workload generated from region j 2 J; Enj  is the set  of states where server n is  the preferred server for  region j; CN   are  the vertices of  a  N-

¼

i

Bi 2 CN : d    ¼ 1

ij

2N

lij PðBi Þþ

i

Bi 2 CN : dþ ¼ 1

ij

kij PðBi Þ   j ¼ 1; .. . ; 2

ð5Þ

dimensional hypercube; d    ; dþ are  the ‘‘downward’’ and ‘‘upward’’

XPðB Þ¼ 1                                                                                            ð6Þ

ij       ij

i

i¼1

Hamming distances between vertices Bi and Bj,   d     þ dþ ¼ dij

and

ij            ij

yl          0; 1g  i 2 I; j 2 J; l ¼ 1; .. . ; N                                                         ð7Þ

kij, lij  are  the upward and downward mean rates at which transi- tions are  made from state i to  state j, corresponding to  vertices Bi and Bj, given that the system is in state i. Finally, we  have the deci- sion  variables:

ij 2f

xi P yl     i 2 I; j 2 J; l ¼ 1; .. . ; N                                                               ð8Þ

ij

N

Xyl

ij ¼ 1  i 2 I; j 2 J                                                                               ð9Þ

xi ¼

1    if we  locate a vehicle at potential site  i

0    otherwise

l¼1

I

Xyl

ij ¼ 1  j 2 J; l ¼ 1; .. . ; N                                                                 ð10Þ

l

y

ij  ¼

1    if vehicle located at  i has  priority l to zone  j

0    otherwise

i¼1

Formally, the  presented model corresponds to an NP-Hard problem

(Geroliminis et al., 2009). It is a non-linear mixed integer program-

The following are  auxiliary decision variables: qnj  is the  fraction of dispatches sending unit n to region j, n = 1, 2, ... , N; P(Bk) is the  stea- dy-state   probability   of    state   corresponding   to    vertex   Bk, k = 1, 2, .. . ,2N.

Eq. (1)  is  the objective function, Mean Response Time  (MRT); constraint (2)  determines the number of  servers to  be  located and constraint (3) is the integrality constraint for the decision var- iable xi. Constraint (4) calculates the fraction of all dispatches that send server n  to  region j using standard queueing theory argu- ments and assuming a zero-line capacity system (calls that arrive when all the servers are  busy are  lost); Eq. (5)  represents detailed balance equations determining steady-state probabilities of the ﬁ-

ming model that has  embedded a queuing sub-model correspond- ing  to  the   ﬁnite-state  continuous  time Markov process. Given  a particular set  of locations for the  servers available and  a preference list for each demand zone with respect to the  same servers, it is nec- essary to ﬁrst solve  the  ﬂow  balance equations given by (5), (6), be- fore  being able   to  calculate the   value of  the   objective function. Although compactly  written,  those equations are   neither  easily determined nor  easily solved. The  number of  ﬂow  balance equa- tions equals 2N, therefore the  number of equations to solve  the  sub- problem grows exponentially with respect to the  number of servers. It has  been mentioned by Galvao  and  Morabito (2008) that in fact the  computer time required to generate the  coefﬁcients of the  linear

system may  be even higher than the  time required to solve  it. That is because of the  complex relationships imposed by  the  combined location and  dispatching decisions. The ﬂow  balance equations lead to  a linear system of equations, whose exact solution requires the calculation of the  inverse for the  matrix of coefﬁcients. The size  of this  matrix grows exponentially, therefore the  time that it  takes to  perform a single iteration to  evaluate a candidate solution can be  prohibitive.

by  using q = 0.1, 0.5, 0.9.  The  server’s speed is  assumed to  be  1.0 distance-units/time. The  maximum threshold used for  coverage was  7.0 distance units. 100  different set  of locations are  generated, and since for  each one  of them three scenarios are  considered for the service rates, we  generate 300  different instances.

One   of  such randomly generated  problem (Table   1)  and its respective optimal solution for  MRT (Table  2)  after enumeration is detailed next. It is worth noticing the number of possible solu-

It was  mentioned that maximizing coverage is the most com-

monly used approach to  planning EMS  systems. Instead of  the

tions: 77,760. There are

5

3

¼ 10

possible locations. Each  de-

standard  coverage, we  use  the concept of  expected coverage as presented by Ingolfsson, Budge,  and Erkut (2008), which takes into account the congestion of the system and potentially the variabil- ity in responses times. The following equations details how to cal- culate the expected coverage:

J             N                                         n   1

Y

mand zone has  an  ordered list  of the servers, and since there are three servers, each customer can  have 3! unique lists.  The  total number of solutions is then 10     3!5 = 77,760. Note  that the inclu- sion  of another demand zone would increase the number of solu- tions to 466,560. In other words, the number of possible solutions increase by  a factor of 3!.  Hence, the search space for  a real  size

Ex: Cov ¼ Xfj XPj;j  n  ð1   P

ð   Þ

jðnÞ Þ

PjðuÞ

ð11Þ

problem is huge, and enumeration is no longer an alternative.

j¼1

n¼1

u¼1

In  Table  2,  St.  Cov refers to  the basic calculation of  coverage,

hence each demand zone is considered covered if there is at least

where Pj,i   is  the  probability that station i covers node j, Pi  corre-

sponds to  the  busy probability of the  ambulance in  station i and j(n) refers to the  nth preferred station for demand node j. Note  that Pj,i  can  be  used as a binary variable, indicating whether or not  the coverage threshold is satisﬁed by  the  available servers, but  it  can also be used as the  probability of that coverage being possible with- in the  given threshold, accounting for variability in travel times. In this  particular case  it has  been used as a binary variable. Eq. (11) re- places Eq. (1) in the  optimization model for the  cases in which we are  looking at  maximum expected coverage.

4. Toy case study

In this section we  introduce a case  study that is small enough that we  can  enumerate all  the possible solutions, also  identifying the optimal. Because of the small size  we  also  use  the exact solu- tion for  the embedded hypercube model. For  this example we use  a  square region on  the cartesian plane and assume that we have ﬁve demand zones, that are  also  candidate locations for three available servers. The locations of the demand zones can  be in any integer ordered pair  within a grid,  starting at (0, 0) and extending up  to  (10, 10)  on  the plane. The  demand for  each zone ranges be- tween 1 and 20  calls/period-time.

To generate different instances we  use  random numbers as fol- lows. The  coordinates (x, y) for  each of the ﬁve  demand zones are obtained by  generating uniform integers between 0  and 10.  For each one  of the demand zones the demand is obtained by generat- ing uniform integers between 1 and 20. The distances between de- mand zones correspond to right angle distances. Optimal locations are nevertheless insensitive to the choice of a distance metric (Ben- veniste, 1985). The service rate for the servers is assigned based on assuming a particular value for  the overall utilization of the sys-

tem, namely q = k/3l. As in the works by Budge,  Ingolfsson, and Er-

kut  (2009) and Chiyoshi et al. (2001),  where q is varied between

0.1  and 0.9,  three different scenarios of  utilization are  evaluated for  each combination of location of demand zones and demands,

one  ambulance located at a distance of 7.0  or  less  distance units; St. Cov = 1.0 means full coverage. However, this deﬁnition of cover- age does not  take into account the congestion of the system, hence we  used Ex. Cov (Eq. (11)).  MRT (Eq. (1))  is the mean system re- sponse time. The  5th column corresponds to  the  probability of the system being busy (all  the servers are  attending calls),   and therefore new calls  would be  rejected. The  last  column indicates the optimal locations of the servers.

At  ﬁrst sight results in  Table  2  correspond to  what was   ex- pected. On one  hand, the increase of the overall utilization, which basically means reducing the service rates while keeping the call rates constant,  causes an  increase in  the expected response time, as well  as an  increase in the busy probability. On the other hand, we  can  see  that the standard coverage is not  able  to  take into ac- count the congestion phenomena. The  expected  coverage given by (11)  is clearly affected by the increase of the overall utilization. The more congested the system, the lower the expected coverage.

We  have solved by enumeration a total of 300  small size  prob- lems, for  both minimum response time and maximum expected coverage. As expected according to  the arguments expressed in Section 2, for  each one  of the 300  problems, the optimal solution that minimizes MRT was  the same as  a solution where the loca- tions were optimized and a  dispatching list  based on  the closest vehicle was  used. We  have also  observed that, when there are  ties (several servers are  at the exactly same minimum distance from a given demand zone), only  one  of  the combinations leads to  the optimal solution, although the use  of  other dispatching ranking, which would still  be based on the closest rule (because of the ties), causes an increase on the objective function value that in the worse case  is below 2%. Note  that the change of the preference list  of a single demand zone, even if for that demand zone several servers’ locations are  tied, changes the overall performance indicators of the system.

Next  we  looked for  maximum expected coverage (Ex. Cov), as given by Eq. (11).  Once  again, we  enumerated all the possible solu- tions for all 300  instances to be able  to identify the ones that gen-

Table 1

Spatial locations and demand - one random instance.

Locations

Table 2

| Optimal (MRT) solution information. Index | x-Coord | y-Coord | Demand |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Performance indicator |  |  |  |  |  |  |  |  |  |  |
| 1 2 | 10 1 | 5 1 | 20 18 |  | q | St. Cov. | Ex. Cov. | MRT | P[1 1 1] | Optimal locations |
| 3 | 7 | 9 | 12 |  | 0.1 | 1.0 | 0.954 | 2.123 | 0.003 | 1–2–3 |
| 4 | 2 | 7 | 8 |  | 0.5 | 1.0 | 0.721 | 4.340 | 0.134 | 1–2–3 |
| 5 | 6 | 1 | 6 |  | 0.9 | 1.0 | 0.517 | 5.355 | 0.309 | 1–2–4 |

optimal solutions do  not  follow the closest dispatching rule. We have also  observed that there are  several solutions that exhibit the same maximum coverage for  a  particular instance, and that the associated response time of those solutions present great var- iation. Since  minimizing the response time is  also  important, in cases where there were several optimal solutions with respect to coverage we  have selected the one  with the minimum associated response time. Although the optimal solutions with respect to ex- pected coverage do not  follow a myopic allocation policy we  have also  noticed that the use  of such a policy would cause a decrease in expected coverage that in all cases is below 3.8%. In fact  for half  of the cases it is below 1.0%.

Since  both objectives are  important, we  compare the optimal solutions obtained with each optimization criteria. We noticed that for  every instance of the problem, the maximum coverage identi- ﬁed  is in fact  greater than the coverage associated with the mini- mum response time solution. However, the increase in  coverage is small on  average, ranging from 0.18% to  17.4%, with a mean in- crease of 3.15% (95% CI: 2.75–3.55%). On  the other hand, the in- crease in  coverage (obtained by  changing the objective function) comes as  a  result of  worsening the response time. The  increase in MRT ranges from 1.5% to  117%, averaging 65.2  (95% CI: 56.33–

74.24%)%. As expected, there is a trade-off between response time and expected coverage. These results seem to  indicate that focus- ing  on  minimizing the response time yields solutions that are  ro- bust with respect to  the expected coverage. There were only   4 cases (out of 300)  in which the proportional improvement in cov- erage (when maximizing coverage) was  in fact higher than the cor- responding increase in  response time. In  all  the other cases the trade-off between improved coverage but increased response time is not  appealing. These results are  aligned with those reported by Geroliminis et al.  (2009),  who mentioned that the optimal loca- tions obtained by using MCLP (a coverage maximization approach) performed up to 40% worse when the response time was  evaluated by using the hypercube model. In the next section we  introduce an optimization framework that allows us  to  solve bigger size  prob- lems, hence allowing us  to  check if the observed behavior of the small instances holds for more real-world sized problems.

5. Genetic Algorithm based optimization framework

Next  we  develop an  optimization framework to solve the com- bined location and dispatching decision problem for EMS systems. The  optimization is based on  GAs. In his  review, Goldberg (2004) suggest that the use  of spatial queuing (hypercube model) or sim- ulation procedures embedded within a heuristic search offers the greatest utility for  real   world EMS planning applications.  Aytug and Saydam (2002) also  comment on the success of GAs in solving combinatorial problems, which make them strong candidates to solve the  ambulance location/allocation problem. Iannoni and Morabito (2007) as  well  as  Iannoni et al.  (2008) and Geroliminis et al. (2011) have also embedded the hypercube model into genetic algorithms to solve the location problem. As mentioned by Geroli- minis et al. (2009) the objective function MRT, as a function of the location space, has  many local  minima, making it suitable for a glo- bal   search procedure such as  GAs.  Jia,  Ordóñez, and  Dessouky (2007b) proposed a GA to solve the problem of locating facilities to  attend large scale emergencies. Shariff,  Moin,  and Omar (2012) used a GA for solving the MCLP problem applied to healthcare facil- ity  location in Malaysia.

Genetic Algorithms were ﬁrst introduced by Holland (1975) and popularized later by  (Goldberg, 1989). GAs are  general-purpose, population based search algorithms that  resemble the  natural selections survival of the ﬁttest. Particular coded schemes (solu- tions representations)  corresponding to  chromosomes represent

evaluated and assigned a  ﬁtness value (related to  the objective function being optimized). According to  their ﬁtness values, solu- tions are  selected to construct the next generation by applying ge- netic   operators:   selection,  crossover  and   mutation.   Current members of  the population are   probabilistically selected based on  their ﬁtness values, where a high ﬁtness value yields a higher chance of being selected for  the next generation. After  selection, current solutions may be  carried to  the next generation without altering (selection), or  they may be  crossed-over to  generate the next set  of solutions. Crossover is an  operator by which two solu- tions mutually interchange their current genes. Mutation is  an operator that randomly alters the value of a gene of a selected solu- tion. Following the work by  Aytug  and Saydam (2002),  there are ﬁve key issues in designing a GA algorithm: (1) Selecting an appro- priate solution representation, (2)  an  effective mutation operator, (3)   an   effective crossover  operator,  (4)   a  feasible  initialization and (5) appropriate crossover and mutation rates as well  as popu- lation size.

5.1. Solution  representation

The  present work uses the idea of  a  composite chromosome. That  is, a chromosome that is in fact  composed of several chromo- somes. This  representation makes sense given the nature of  the problem in  which there are  two decisions to  be  made, a location decision and a dispatching decision. Therefore, those two decisions are  coded in  separate sub-chromosomes. Furthermore, since the dispatching decision is in fact  one  decision per  each demand zone, that gives  rise  to the idea of having separate chromosomes to rep- resent each demand zone. Fig. 2 shows the composite chromosome for a case  in which there are  three servers to be located among ﬁve candidate locations to  attend ﬁve  demand zones (every demand node is a candidate to locate a server). Note  that the chromosome has  been divided into sub-chromosomes. The  ﬁrst one  deals with the location decision, and therefore has  size  5, with the three ﬁrst components storing the location of a server. The location sub-chro- mosome stores more information than required, since only  a sub- set  of locations will  have a server. However, it is kept that way  to facilitate feasibility checking as well  as the mutation operation, de- scribed later. The remaining sub-chromosomes have size  3, repre- senting the order in  which every server is  ranked to  attend  a particular demand zone. Note  that the sub-chromosome for  the location decision corresponds to  a  permutation of  the candidate locations and any  sub-chromosome for  the dispatching decision corresponds to a permutation of rankings.

5.2. Mutation operator

The  standard mutation operator randomly selects a  chromo- some from the pool,  and then goes  through every one  of its  genes changing them randomly with a  given probability. Since  we  are using a composite chromosome, once a chromosome has  been se- lected for  mutation the operation should analyze every one  of its sub-chromosomes.  For  we   are   working with  sub-chromosomes that are  a permutation, the standard mutation operator is replaced by a swapping operator. It randomly interchanges the positions of two genes within the chromosome, as depicted in Fig. 1. Note  that for  the location sub-chromosome the swap is done such that the interchange occurs between an  assigned location in  the current

Fig.  1.  Swapping mutation operator.

Fig.  2.  Composite chromosome.

solution, and a candidate location not  yet  selected. That  is in order to  avoid swaps that do  not  affect the solution.

5.3. Cross-over operation

A single point cross-over operation is used on the implementa- tion of the GA. The recombination of genes is done at sub-chromo- somes level,   which means that  the candidate cross-over points correspond to  sub-chromosomes  as  well.   To  better understand the way   it  operates an  illustrative example is  given in  Fig. 3.  A and B are  the two parents. O1 and O2 represent the two offsprings that it is possible to generate. Parent A has  been shadowed so that it is possible to trace where the genes of it are  going to be after the cross-over operation. The possible crossover points are represented by vertical dashed lines.

5.4. Population initialization

It  is  usually the case  that the initialization is  done randomly. The existence of constraints might require us to develop initializa- tion routines that produce feasible solutions. In this case  the pop- ulation  of   the  GA   can    be    randomly   initialized,  since  any permutation for   any   sub-chromosome  will   generate a  feasible solution. However, it is also  possible to use  an initialization proce- dure to  create ‘good’  initial solutions, using the available knowl- edge about the problem. Initial tests of  the GA implementation were done with a random generated population. Based  on  the re- sult from the enumeration procedure for the small case  study pre- sented in  Section 4,  a  better initialization procedure is  devised. Since  the use  of the closest dispatching rule seems to  effectively helps in minimizing the response time also  providing good  cover- age, it makes sense to use  that information as part of the initializa- tion process. In fact, when solving mid-size problems, the locations are  generated randomly during the initialization, but the dispatch- ing  is based on  the use  of the closest servers ﬁrst.

5.5. Cross-over and  mutation rates – population size

In order to  test the performance of the GA values of mutation (Pm) and cross-over (Pc)  rates are  required. Iannoni et al.  (2008) used Pc = 0.5,  and Pm = 0.06,  while the population size  was  set  to S = 100  individuals. In  turn, Aytug  and Saydam (2002) suggested Pc = 0.6 and Pm = 0.03,  while the population size  was  set  according to  S = max (100;0.75n), where n  is  the number of  nodes in  the problem being solved. The  authors argued that for objective func- tions with potential multiple local  optima, there is a trade-off be- tween mutation and cross-over, and that large population sizes

are  generally favorable, at the cost  of computation time. It is also mentioned that the rule of thumb Pm = 1/L,  where L refers to  the length of  the  chromosome could yield good   results.  Instead  of selecting arbitrarily values for  these GA parameters, in  the next section we introduce an experimental design to tune-in the param- eters of the GA before using it. The  implementation of the GA has been done using the Java  GA framework developed by  Meffert, Meseguer, DMartf, Jerry,  and Rotstan (2012).

6. Computational results

6.1. Tuning  the  GA

A tuning procedure was  carried out  to ﬁnd  adequate values for several parameters of the GA. The purpose of any  experiment is to get   the  maximum amount  of  information with  the  minimum expenditure of  resources. A Central Composite Experimental  De- sign  (CCD) was  used, which according to  Montgomery (2008) is widely used because it is highly efﬁcient and ﬂexible. A CCD is nor- mally used to ﬁt a second order polynomial model of a variable of interest. In our  case  we  are  not  trying to  ﬁt  a polynomial model. However the  combination of  factors’ values  suggested  by   the CCD provide a good  exploration of trade-offs between the different parameters of the GA and its  general performance.

There are  three parameters that need to  be  set  up: mutation rate, cross-over rate and population size.  In  experimental design the parameters are  called factors, and for  each one  of these three factors it is necessary to specify a minimum and a maximum value. The minimum and maximum values to be tested for each parame- ter have been selected according to  general recommendations  of designing GAs from previous works (Aytug & Saydam, 2002; Ian- noni et al., 2008). The CCD also  uses the midpoint of the factor (gi- ven  the minimum and maximum values), as well  as the so-called axial  points. Axial  points correspond to  values of the factors that assure that the predicted values of the ﬁtted response surface have the same variance, if the predicted points are  at the same distance from the center of the design region (Montgomery, 2008). For the case   of  three factors a  standard CCD requires 20  runs. The  ﬁrst

14 runs correspond to different combinations of the factor’s levels, while the last  6 runs correspond to experiments in which each fac- tor  is set  to its midpoint. A standard CCD does not  uses replication. We do use  it (30  runs for each combination of factors), as a way  to improve the statistical signiﬁcance of the tests. Instead of the 6 last runs each with one  replication, we  have a  single run setting the factors to their midpoints and we  replicate it 30 times. We explore

15  combinations of factors, detailed in Table  3.

Fig.  3.  Single point cross-over for  the composite chromosomes.

Combination of factors for  experimental  design. 	 Combination                              Pm                                             Pc                                                        Pop. size

1                                         0.02                          0.4                                      30

2                                         0.05                          0.4                                      30

3                                         0.02                          0.6                                      30

4                                         0.05                          0.6                                      30

5                                         0.02                          0.4                                    100

6                                         0.05                          0.4                                    100

7                                         0.02                          0.6                                    100

8                                         0.05                          0.6                                    100

9                                         0.01                          0.5                                      65

10                                         0.059                        0.5                                      65

11                                        0.036                       0.332                         65

12                                        0.036                       0.668                         65

13                                        0.036                       0.5                                         6

14                                        0.036                       0.5                                    123

15                                        0.036                       0.5                                      65

The  minimum and maximum values for  the mutation rate are

2% and 5% (the values used in  the experiments are   then those two, plus the center point, 3.5% and the axial  points 1% and 6%). For  the Cross-over the minimum and maximum values are  40% and 60% and the population size  varies between 30  and 100  indi- viduals. In  all  the runs of  the GA while tuning the parameters, the number of evolutions is set  up so that the total number of indi- viduals being evaluated remains constant (approximately equal to

10.000). For example, if the population size  is set  to  30  then 334 evolutions are  performed.

The results from the tuning procedure are  given by the box plot graph shown in Fig. 4. It corresponds to the tuning for MRT optimi- zation. As it was  mentioned before, for each combination of factors given in Table  3, the GA was  run 30 times, applied to different in- stances and each time using a different random seed. In each case

100   evolutions of  the GA were allowed. We  have noticed that allowing more evolutions did not  further improve the objective va- lue.  In order to  have a comparison point to  tune the GA we  enu-

merate   only     the   location   solutions   for     the   case     study

means that the GA was  able  to ﬁnd  a solution with the same objec- tive  function value. Positives values of the Gap would indicate that the combination of dispatching and location decisions was  useful in  getting a  better value for  the objective function. Recall  that the gap  reported is the average over  30 runs. Out  of the 15 combi- nations of  factor’s levels under consideration, combinations 7,  8 and 14  showed the best overall performance. All have a Gap  close to  zero,  and exhibited low  variability. We  ran  normality tests on the selected combinations and could not  verify the normality of the data. Therefore, we  performed a non-parametric test, the Wil- coxon Signed Rank Test, to obtain the Conﬁdence Intervals (CIs) for the three candidate combinations. Table  4 shows the results from the non-parametric test. As it can be seen, the overlapping CIs indi- cate that statistically there is no  difference between the parame- ters combinations. We  decided to  use  combination 8  which has the smaller CI.

6.2. Mid-size  case  study

We  solved a bigger, mid-size problem, proposed as an  instance of MCLP (http://www.lac.inpe.br/lorena/correa/Q_MCLP_30.txt) (Correa, Chaves, & Lorena, 2007). We  analyze several scenarios, varying the number of servers to  be  located, considering 3 and 4 ambulances. Because of the small number of ambulances we  use again an  exact solution for the hypercube model. The  server rates are  obtained by  selecting particular values for  the overall utiliza-

tion factor, q = k/(N     l). In fact,  q is varied between 0.1  and 0.9,

with increases of  0.1.  For  the scenarios having three servers we use  full  backup, which means that any  zone can  be  attended by any  of the available servers. In the case  of four  servers we  use  par- tial  backup, therefore each demand zone is  only   allowed to  be served by 3 of the available servers. There are  two reasons to pro- ceed this way,  that have also  been suggested by Geroliminis et al. (2009): (i)  from a practical perspective, allowing servers that are ranked as 4th and up for a particular demand zone is not  desirable,

30

3

¼ 4060

possible location decisions. It is not  possible to

because the overall efﬁciency of the system would likely  decrease;

(ii) the calculation of transition rates for the embedded hypercube model becomes very  tedious.

also  enumerate the dispatching decisions, which would be compu-

tationally prohibitive. For  each possible location solution we  use the closest rule to set  the priority dispatching list  of each demand zone. We then compare the performance of the GA (GASol) against the best solution found (BestSol) following the enumeration proce- dure  just  described.  The    Gap   is   calculated  as   (Gap = (Best- Sol     GASol)/BestSol).

Negative values of the Gap indicate that the GA obtained a solu- tion with a worse objective function value than the best solution

For  each instance we  ran  the GA using the tuned parameters and initially minimizing the MRT. In each case  we  also  enumerate the location solutions, and combine them with the use  of the clos- est  dispatching policy to have a full solution. That  gives  us a com- parison point. The  GA was  allowed to  run for  100  evolutions. We have noticed that allowing more evolutions does not  improve the results. The performance of the GA is compared to the best solution coming from the enumeration procedure. Table 5 shows the results of applying the GA, in each case  running it 30  times starting with different initial solutions. The  experiments have been run on  a PC executing Windows 7-64 Bit, with an Intel    Core 2 Duo proces- sor  running at 2.13  GHz  and 2 GB of  RAM. All the programming was  done in Java. The average running time of the GA for the three servers scenarios was  20 s, while the average for  the case  of four servers was  55 s.

As expected given the low-medium trafﬁc (Jarvis,  1981; Kateha- kis  & Levine,  1986), for  the mid-size problem the results suggest

Table 4

Wilcoxon test for  obtaining  CIs.

Conf.  interval

| Fig. 4. Results of GA experimental design tuning for MRT minimization. Combination | Median gap (%) | Lower | Upper |
| --- | --- | --- | --- |
| Comb7 | 0.191 | 0.823 | 0.058 |
| Comb8 | 0.208 | 0.476 | 0.049 |
| Comb14 | 0.271 | 0.712 | 0.109 |

Table 5

| Mid-size case study – MRT results. q | MRT | Gap (%) | CV |  | MRT | Gap (%) | CV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 0.587 | 0.26 | 0.0039 |  | 0.485 | 0.51 | 0.0054 |
| 0.2 | 0.649 | 0.07 | 0.0015 |  | 0.518 | 0.27 | 0.0043 |
| 0.3 | 0.679 | 0.08 | 0.0022 |  | 0.536 | 0.37 | 0.0050 |
| 0.4 | 0.703 | 0.02 | 0.0009 |  | 0.542 | 0.53 | 0.0041 |
| 0.5 | 0.722 | 0.02 | 0.0001 |  | 0.544 | 0.78 | 0.0068 |
| 0.6 | 0.737 | 0.03 | 0.0012 |  | 0.551 | 0.46 | 0.0071 |
| 0.7 | 0.750 | 0.14 | 0.0054 |  | 0.591 | 1.46 | 0.0160 |
| 0.8 | 0.759 | 0.04 | 0.0014 |  | 0.602 | 0.98 | 0.0117 |
| 0.9 | 0.767 | 0.00 | 0.0000 |  | 0.621 | 1.00 | 0.0113 |

3 Servers scenarios                            4 Servers scenarios

Table 6

Hanover case study – MRT and exp. coverage.

Min.                     Gap                            Exp.                          Max.

q                      MRT                         MRT (%)                        Cov.                         Ex. Cov.

0.2                         4.53                       1.48                       0.88                    0.89

0.3                         4.82

0.4                         4.97

1.08                        0.83                    0.85

1.03                        0.78                    0.81

that a  policy that focuses on  appropriately selecting locations in combination with dispatching the closest server minimizes mean system response time. These results serve as  a  validation of  the general structure of the mathematical model as well  as for the cor- rectness of the optimization procedure. Note  that the MRT is smal- ler when we  have an additional server available. Also, for the same number of servers, increasing the utilization of the system also  in- creases the MRT. For  the mid-size case  studies we  have also  ob- served that the expected coverage associated with the solution that minimizes the MRT is smaller (7.2% smaller, on  average. 95% CI: 6.14–8.31%) than the maximum observed after the enumera- tion procedure.

Next  we  approached the optimization of the system maximiz- ing  the expected coverage. A procedure similar of that described in  Section 6.1  was  followed to  tune the GA to  be  used with the new objective function, Expected Coverage. In this case  the combi- nation 7 (from Table  3) showed the best results and therefore was selected as the values for the GA parameters. The enumeration pro- cedure of the location decisions together with a myopic dispatch- ing   policy  was   used  again  to   identify the  solution with the highest expected coverage. The  performance of the GA was  com- pared against the solution from enumeration. The  overall average Gap   of  the  GA  compared  to   the  enumeration  procedure was

0.87%. The overall mean coefﬁcient of variation of the maximum coverage was  0.0136. These performance measures of the GA show that the algorithm was  consistently able  to get  to the same or to a very  close solution from the best found by the enumeration proce- dure. Compared to the solution that minimizes the response time, the average improvement in coverage is 7.9% (95% CI: 6.64–9.09%). However, this increase comes at a price, a sacriﬁce of MRT that on average increased by 19.2% (95% CI: 16.35–21.97%). Recall  that the expected coverage of the solution minimizing MRT was  on average

7.2% smaller than the maximum obtained with enumeration, while the increase on MRT would be on average 19.2% as a result of max- imizing coverage. The  joint location/allocation approach was  not able  to improve the solution found by combining the enumeration of locations and the closest dispatching policy.

6.3. Hanover County  case  study

Here  we  introduce a case  study using real  data from the Hano- ver  County Fire/EMS  department  (Hanover, VA). The  county has been divided for  planning purposes in  122  demand zones. There are  16  candidate locations for  ﬁve  ambulances. The  total demand rate has  been estimated in  1.2 calls/h. The  average service time per  call  has  been estimated to  be  74 min and it  is assumed to  be independent of the demand zone being served. The system covers

474  square miles and a population nearing 100,000 individuals. For this case  study we use  partial backup, allowing every demand zone to be served only  by three out  of the ﬁve available servers. The uti- lization factor for this real  system is q = 0.2. However we  consider

Table 7

Mid-size case study – workloads and individual MRT.

3 Servers – CV                                             4 Servers – CV

q                 Workload               Ind. MRT                  Workload                Ind. MRT

0.1                   0.387                      0.594                    0.357                      0.605

0.2                   0.250                     0.590                    0.105                     0.529

0.3                   0.039                     0.565                    0.318                     0.489

0.4                   0.029                     0.541                    0.335                     0.436

0.5                   0.038                     0.549                    0.325                     0.512

0.6                   0.018                      0.539                    0.317                      0.501

0.7                   0.031                     0.531                    0.612                     0.556

0.8                   0.028                     0.525                    0.609                     0.551

0.9                   0.025                     0.520                    0.605                     0.548

two variations, increasing the demand by  a  factor of  1.5  and 2 respectively (which increases the overall utilization, q).  For  this case   study we   are   also   using the exact procedure to  solve the hypercube model (with ﬁve  servers the number of  states is  32). The  average running time of the GA for  each scenario of this case study was  280  s. Table  6 shows the results for the Hanover scenar- ios. The gap  reported in Table  6 (third column) is a comparison of the MRT obtained by  using the joint location/allocation approach versus the best solution found by solving a location only  problem for which the dispatching rule is always sending the closest avail- able   vehicle available. As  the gap   values indicate, the joint ap- proach is able  to  produce results for  the MRT that are  around 1% of the same criterion obtained by the location only  approach. The fourth column shows the expected coverage associated with the solution that minimizes the response time, while the last  column shows the maximum possible coverage (obtained by  solving the model with the objective of maximizing expected coverage).

6.4. Non-efﬁciency criteria

Thus  far we have introduced an optimization framework for the joint location/allocation problem, however we  have noticed that for the two most common objectives the joint approach is not  add- ing value, since the use  of a myopic policy seems to sufﬁce to get to the optimal or  near optimal solution. Hence we  have turned our attention to  calculating other performance indicators for  the sys- tem. In  particular,  other works have mentioned the importance of ﬁnding solutions in which the total workload is evenly distrib- uted among the available servers, and some others have mentioned that it  would be  desirable to  have individual response times (the mean response time for  each demand zone) that do  not  vary  too much among the demand zones. Both  performance indicators are associated to the idea of fairness, either from an  internal or exter- nal  point of view. In Table  7 we  present the coefﬁcient of variation (CV) for both, mean individual workloads and mean individual re- sponse times, resulting from the solutions that optimize mean re- sponse time. In this table several instances of high CV values (for example P0.5) are  observed, which implies high variability among server’s workloads or demand zones’ response time.

Among the several instances of the case  study it  is possible to notice that variability on  individual response times tends to  be higher (see  Table  7), hence we  attempted to  improve that perfor-

Mid-size case study – optimizing CV resp. times. 	 Min CV Ind. RT                     Delta                       Trade-offs (%)

q              Enum.            Loc./Disp.           CV (%)                  MRT                  Ex. Cov.

0.1              0.489            0.364                   25.577           37.853             11.980

0.2              0.504            0.355                   29.652           32.423             10.461

0.3              0.496            0.367                   25.948           24.485               4.603

0.4              0.478            0.375                   21.571           19.698               3.981

0.5              0.465            0.380                   18.285           16.345               3.911

0.6              0.458            0.384                   16.154           13.764               3.497

0.7              0.454            0.389                   14.300           11.954               3.472

0.8              0.481            0.392                   18.636           11.051               3.098

0.9              0.476            0.397                   16.534             9.612               3.022

mance indicator by using the optimization approach already devel- oped. Eq. (1)  gives  the total average response time for  the system and in  doing so  it  includes the response time for  each demand zone. We  use  the coefﬁcient of variation (CV) of the individual re- sponse times as  the new optimization criteria. Once  again it  was necessary to  tune the GA with the new objective function. The GA parameters that perform the best were the same as for the case of MRT minimization. We  used the enumeration procedure of the location decisions to  get  a  reference point of  the minimum CV for  the response times and the compare those solutions with the ones obtained by  the joint location/allocation approach. Table  8 shows the results for several instances of the problem, all of them using three servers.

The second column in Table  8 shows the minimum CV for indi- vidual response times that was  attainted by using the enumeration procedure for the location decisions in combination with the clos- est  dispatching rule. The third column shows the CV that was  pos- sible to   achieve by  using the  proposed optimization approach while the  fourth column compares the  two previous, showing the relative improvement that was  possible thanks to the joint ap- proach. The  last  two columns show the sacriﬁces in  MRT and Ex- pected  Coverage that  come  as   a   result  of   the  reduction  in response times variability across demand zones. We see  that there are  both an increase in response time and a reduction in coverage. The  size  of the trade-offs depends upon the utilization (q)  of the system. The trade-offs were calculated using the solution that min- imizes response time as a reference point. For instance, for q = 0.4 there is a reduction of 21.5% in response time variability, as mea- sured by the coefﬁcient of variation, as well  as in increase of about

20% in response time and a reduction of 4% in coverage.

7. Results summary and discussion

We  have done extensive computational experiments using 300 small case   studies (enumerating more that  70,000 solutions for each instance). We  were  looking for  a  better  understanding  of the  potential beneﬁts when location and dispatching decisions are  made together for  an  EMS system. The  instances have been generated randomly therefore not   favoring any  particular result in terms of the decisions being made. Although previous literature had suggested that the existence of demand zones with very  differ- ent demand rates could lead to situations in which the dispatching based on  the closest rule was   not   optimal, our   results were in agreement with some other references showing that using a myo- pic  policy can  lead to  optimal solutions. We  have allowed the de- mand rates  to   vary   between  1  and  20,   therefore  introducing differences in  the demand rates. What we  have found is  that  if the dispatching policies are  designed as a ﬁxed priority list  associ- ated to each demand zone, then focusing on ﬁnding good  locations, and combining them with the use  of the closest dispatching rule, yields the desired result of minimizing the mean response time.

mize in  EMS system, we  have used an  expected version of cover- age,  since previous works have made it  clear that the standard coverage,  which does not  take into account the congestion phe- nomena, overestimates the real  coverage. For the small instances we  have found that the solutions that maximize the coverage did not  use  the dispatching policy based on the closest rule. However, we  have also  noticed that the improved coverage that comes as a result of its  maximization, causes a deterioration in  the mean re- sponse time. As pointed out  in Section 4, optimizing the coverage increases it  less  than 5% (compared to the coverage obtained by the  best solution with respect to   MRT),  while the  sacriﬁce in MRT would be  greater than 60%. Those  results basically suggest that optimizing the MRT is a better strategy, and that in fact the re- sults in  coverage when optimizing MRT are  robust, in  the sense that the coverage is only  1 or 2% below its optimal value. These re- sults about coverage were also  validated with a mid-size real  case study found and adapted from previous literature. For the mid-size cases the best coverage was  reached when using the closest policy. The average improvement in coverage was  again smaller than the average increase in response time.

Results from alternative performance indicators such as  those depicted in  Table  7,  suggest some other observations. Given  the values for  the CVs it  is  not   surprising that in  some cases there are  some demand zones with a  MRT that doubles that of  other zones, or  one  ambulance having a  much heavier workload than the others. Solutions that are good  from the point of view of system wide mean response time, can  have other performance indicators affected negatively. Since  the optimization has  been done with a single objective in mind, there is no guarantee of good  performance with respect to other criteria. Our results have shown that optimiz- ing the MRT also  yields good  values for expected coverage. That  is convenient since those two are  the most common performance indicators used for  planning purposes of EMS systems. We  illus- trated the potential beneﬁts of the joint approach by  considering a  fairness performance indicator from the  user  point  of  view, namely coefﬁcient of  variation for  individual response times. In this case,  the joint approach was  able  to ﬁnd  better solutions than those that could be reached by using a myopic allocation policy. Of course, the improvement of  a  fairness objective like  the one  we have used has  consequences, altering other performance indicators such as MRT and coverage. It would be up to the decision maker to balance those trade-offs.

8. Conclusions

Our  main goal  was  to  develop an  optimization framework for the joint location/allocation problem for  EMS systems. We  com- bined the mathematical model and a heuristic solution procedure based on  Genetic Algorithms, to  be  able  to  solve bigger instances in which enumeration is no longer an option. We were able  to val- idate our  approach. The  GA has  been consistently able  to  ﬁnd  the same or  a pretty close solution to  that obtained by  full  or  partial enumeration  procedures. In  terms of  MRT minimization or  Ex- pected Coverage maximization we have noticed that the integrated approach do  not  offer  tangible beneﬁts. A more simpler approach considering only  the location decision combined with a  myopic allocation  of  the  servers based  on   closest distance  would  be enough.

One  general explanation of the observed behavior is that MRT and Expected Coverage are  in fact  a function of the distance (time) between servers and demand zones. Hence, locations that reduce the overall distance between servers and costumers tend to  dom- inate the optimization procedure. Although in  this case  we  could just have proposed an  optimization procedure in  which the deci-

est  dispatching rule,  we  have kept both sets of decisions as part of the optimization framework. We  believe that it  is  important be- cause it  gives  us  the opportunity to  attempt the optimization of other performance indicators, so that we can see the trade-offs that are  being made as  a  result focusing on  minimizing the response time or maximizing coverage. The fact that solutions that minimize response time offer  at the same time a good  expected coverage is convenient, since those two criteria are  the most commonly used. There is another important consideration: regulations are  usually imposed as coverage thresholds, which leads to coverage maximi- zation as preferred optimization criteria. However our results show that the coverage maximization approach can  lead to  sacriﬁces in response time that do  not  compensate for  the gains achieved in coverage. On the contrary, minimizing response time offers a good trade-off with respect to  the maximum coverage.

We  have illustrated two alternative criteria, in  particular vari- ability on individual response time, as well  as variability on ambu- lances   workloads.   Those    criteria   can    be    seen   as    fairness performance indicators from the perspective of internal and exter- nal  customers. We  used individual response time variability as  a optimization criteria. As in  the case  of  maximizing the expected coverage, when  focused on   reducing response  time  variability among demand zones it is the case  that the best solutions do  not follow the use  of  the closest dispatching rule. Furthermore, the improvements that can  be  made on  variability are  important, and not  only  marginal as in the case  of maximizing coverage. The pro- posed optimization framework, already proven to  work correctly, can  be  used to  analyze the EMS system from other perspectives, gaining insight into the design of better operation strategies.

As future research directions we  will  attempt to  identify other performance indicators of EMS systems for which the joint location and dispatching problem can  yield substantial gains. Another po- tential area for  future research deals with the issue of scalability. We are  aware of the limitations of our  approach in terms of apply- ing  the joint model and its  solution procedure to  real-sized case studies, basically because the exact solution of the hypercube mod- el  will  likely  require extensive computation time (recall that the exact solution to  the hypercube model requires solving a  linear system of equations that grows exponentially in size  with respect to the number of servers available in the system). However, avail- able  approximation procedures that have been suggested in the lit- erature  could be  embedded in  the meta-heuristic optimization framework proposed, hence reducing the computational burden and allowing the solution of bigger instances.